🌐 CCNA Lab 10 – Route Redistribution Between OSPF and EIGRP

📌 Lab Title

Redistribution Between OSPF Process ID 1 and EIGRP AS 50

In this version, the protocols are swapped:

- Left Side = OSPF Process ID 1

- Right Side = EIGRP AS 50

- R2 = Redistribution Router

🎯 Lab Objective

The objective of this lab was to configure route redistribution between two different dynamic routing protocols:

Left Side

OSPF Process ID 1

Right Side

EIGRP AS 50

The goal achieved:

✅ Configure OSPF Routing

✅ Configure EIGRP Routing

✅ Configure Redistribution on R2

✅ Exchange Routes Between OSPF and EIGRP

✅ Verify External Routes

✅ Successfully Ping Between Different LANs

📖 What is Route Redistribution?

Route Redistribution allows routers running different routing protocols to exchange routing information.

Example:

OSPF Network

│

▼

Redistribution Router

│

▼

EIGRP Network

Without Redistribution:

OSPF knows only OSPF routes

EIGRP knows only EIGRP routes

With Redistribution:

OSPF routes become visible to EIGRP

EIGRP routes become visible to OSPF

🛠️ Devices Used

| Device | Quantity |
| --- | --- |
| Cisco 2901 Routers | 3 |
| Cisco 2960 Switches | 2 |
| PCs | 2 |

📊 Network Topology

OSPF DOMAIN

PC1

|

SW1

|

R1 -------- R2 -------- R3

192.168.1.0   192.168.2.0

EIGRP 50

|

SW2

|

PC2

📋 IP Addressing Scheme

LAN 1 (OSPF Side)

| Device | IP Address |
| --- | --- |
| PC1 | 192.168.3.2 |
| R1 G0/1 | 192.168.3.1 |

Network:

192.168.3.0/24

R1 – R2 Link

| Device | IP Address |
| --- | --- |
| R1 G0/0 | 192.168.1.1 |
| R2 G0/0 | 192.168.1.2 |

Network:

192.168.1.0/24

R2 – R3 Link

| Device | IP Address |
| --- | --- |
| R2 G0/1 | 192.168.2.1 |
| R3 G0/0 | 192.168.2.2 |

Network:

192.168.2.0/24

LAN 2 (EIGRP Side)

| Device | IP Address |
| --- | --- |
| PC2 | 192.168.4.2 |
| R3 G0/1 | 192.168.4.1 |

Network:

192.168.4.0/24

🔹 Step 1: Configure OSPF on Left Side

R1

router ospf 1

network 192.168.1.0 0.0.0.255 area 0

network 192.168.3.0 0.0.0.255 area 0

R2

router ospf 1

network 192.168.1.0 0.0.0.255 area 0

🔹 Step 2: Configure EIGRP AS 50 on Right Side

R3

router eigrp 50

no auto-summary

network 192.168.2.0 0.0.0.255

network 192.168.4.0 0.0.0.255

R2

router eigrp 50

no auto-summary

network 192.168.2.0 0.0.0.255

🔹 Step 3: Configure Redistribution on R2

R2 is connected to both routing domains.

Redistribute OSPF into EIGRP

router eigrp 50

redistribute ospf 1 metric 10000 100 255 1 1500

Why Metric Is Required?

EIGRP requires:

Bandwidth

Delay

Reliability

Load

MTU

Example:

10000 100 255 1 1500

Redistribute EIGRP into OSPF

router ospf 1

redistribute eigrp 50 subnets

Purpose

Imports EIGRP routes into OSPF.

The keyword:

subnets

ensures all subnet routes are advertised.

🔹 Step 4: Verify Neighbor Relationships

OSPF

show ip ospf neighbor

Expected:

Neighbor State FULL

EIGRP

show ip eigrp neighbors

Expected:

Neighbor learned successfully

🔹 Step 5: Verify Routing Tables

On R1 (OSPF Side)

show ip route

Expected:

O E2 192.168.2.0/24

O E2 192.168.4.0/24

Meaning

O E2 = External OSPF Route

These routes originated from EIGRP.

On R3 (EIGRP Side)

show ip route

Expected:

D EX 192.168.1.0/24

D EX 192.168.3.0/24

Meaning

D EX = External EIGRP Route

These routes originated from OSPF.

🔹 Step 6: Test Connectivity

From PC1 (OSPF LAN)

ping 192.168.4.2

Result:

!!!!!

Successful.

From PC2 (EIGRP LAN)

ping 192.168.3.2

Result:

!!!!!

Successful.

Packet Flow

PC1 (192.168.3.2)

│

▼

R1 (OSPF)

│

▼

R2 (Redistribution Router)

│

▼

R3 (EIGRP 50)

│

▼

PC2 (192.168.4.2)

🔹 Step 7: Save Configuration

copy running-config startup-config

Run on all routers.

📚 Verification Commands

show ip route

show ip protocols

show ip ospf neighbor

show ip eigrp neighbors

show ip route ospf

show ip route eigrp

show running-config

ping

🎓 Concepts Learned

✅ OSPF Process ID 1 Configuration

✅ EIGRP AS 50 Configuration

✅ Route Redistribution

✅ OSPF External Routes (O E2)

✅ EIGRP External Routes (D EX)

✅ ASBR Configuration

✅ Routing Table Verification

✅ End-to-End Connectivity Testing

📝 Lab Summary

In this lab, OSPF Process ID 1 was configured on the left side of the network while EIGRP AS 50 was configured on the right side. Router R2 acted as the redistribution router and exchanged routing information between both routing protocols. OSPF routes were redistributed into EIGRP using the redistribute ospf 1 metric command, and EIGRP routes were redistributed into OSPF using the redistribute eigrp 50 subnets command. After verifying the routing tables, OSPF routers learned EIGRP networks as O E2 routes, and EIGRP routers learned OSPF networks as D EX routes. The lab objective was successfully achieved when PC1 (192.168.3.2) in the OSPF LAN successfully pinged PC2 (192.168.4.2) in the EIGRP LAN, confirming successful route redistribution and end-to-end communication between the two different routing domains.