🌐 CCNA Lab 06 – EIGRP Routing (AS 100)

📖 Introduction to EIGRP

What is EIGRP?

EIGRP (Enhanced Interior Gateway Routing Protocol) is an advanced dynamic routing protocol developed by Cisco. It allows routers to automatically share network information with each other and learn routes dynamically.

Unlike Static Routing, where administrators manually configure routes, EIGRP automatically discovers and updates routes when network changes occur.

Why Use EIGRP?

Suppose your network contains many routers:

R1 ---- R2 ---- R3 ---- R4

If Static Routing is used, every router must be manually configured with routes to all remote networks.

With EIGRP:

- Routers automatically exchange routing information.

- New networks are learned automatically.

- Route updates occur automatically.

- Network administration becomes easier.

Features of EIGRP

1. Fast Convergence

When a link fails, EIGRP quickly finds an alternate path.

2. Supports VLSM and CIDR

EIGRP supports modern subnetting techniques.

3. Uses DUAL Algorithm

DUAL stands for:

Diffusing Update Algorithm

This algorithm helps EIGRP select the best route and backup route.

4. Low Bandwidth Usage

EIGRP sends updates only when network changes occur.

5. Load Balancing

Traffic can be distributed across multiple equal-cost paths.

Important EIGRP Terms

Neighbor

Routers directly connected and running EIGRP become neighbors.

Example:

R1 -------- R2

R1 and R2 become EIGRP neighbors.

Successor Route

The best route selected by EIGRP.

Example:

R1 → R2 → R4

This becomes the Successor Route.

Feasible Successor

Backup route stored in the topology table.

Example:

R1 → R3 → R4

If the primary route fails, EIGRP immediately uses the backup route.

Administrative Distance

EIGRP Internal Route:

90

EIGRP External Route:

170

Lab Topology

We used the same topology as Lab 05.

R2

192.168.2.0/24

/        \

/          \

/            \

/              \

R1                R4

\              /

\            /

\          /

\        /

192.168.3.0/24

R3

LAN 1                                 LAN 2

192.168.1.0/24                    192.168.6.0/24

Router IDs Used

| Router | Router ID |
| --- | --- |
| R1 | 1.1.1.1 |
| R2 | 2.2.2.2 |
| R3 | 3.3.3.3 |
| R4 | 4.4.4.4 |

Configuration Performed in This Lab

Step 1: Basic Router Configuration

First, basic security settings were configured on every router.

Configure Hostname

Example on R1:

enable

configure terminal

hostname R1

Purpose:

- Identifies the router.

- Makes troubleshooting easier.

Configure Enable Secret

enable secret cisco123

Purpose:

- Protects privileged EXEC mode.

- Encrypts the password.

Configure Console Password

line console 0

password console123

login

exit

Purpose:

- Secures physical console access.

Encrypt Passwords

service password-encryption

Purpose:

- Encrypts passwords stored in the configuration.

Step 2: Configure IP Addresses

Each router interface received an IP address.

R1

interface fa0/0

ip address 192.168.1.1 255.255.255.0

no shutdown

LAN interface connected to Switch 1.

interface s0/0/0

ip address 192.168.2.1 255.255.255.0

clock rate 64000

no shutdown

Connection to R2.

interface s0/0/1

ip address 192.168.3.1 255.255.255.0

clock rate 64000

no shutdown

Connection to R3.

R2

interface s0/0/0

ip address 192.168.2.2 255.255.255.0

no shutdown

Connected to R1.

interface s0/0/1

ip address 192.168.4.1 255.255.255.0

no shutdown

Connected to R4.

R3

interface s0/0/0

ip address 192.168.3.2 255.255.255.0

no shutdown

Connected to R1.

interface s0/0/1

ip address 192.168.5.1 255.255.255.0

no shutdown

Connected to R4.

R4

interface fa0/0

ip address 192.168.6.1 255.255.255.0

no shutdown

LAN interface connected to Switch 2.

interface s0/0/0

ip address 192.168.5.2 255.255.255.0

no shutdown

Connected to R3.

interface s0/0/1

ip address 192.168.4.2 255.255.255.0

no shutdown

Connected to R2.

Step 3: Configure EIGRP Autonomous System 100

EIGRP was enabled using AS Number 100.

R1

router eigrp 100

Meaning:

Start EIGRP process using AS 100.

All neighboring routers must use the same AS number.

Advertise Networks

network 192.168.1.0 0.0.0.255

network 192.168.2.0 0.0.0.255

network 192.168.3.0 0.0.0.255

Purpose:

- Advertises directly connected networks.

- Allows EIGRP packets to be sent on these interfaces.

Configure Router ID

eigrp router-id 1.1.1.1

Purpose:

- Uniquely identifies R1 in the EIGRP domain.

Disable Auto Summary

no auto-summary

Purpose:

- Prevents automatic summarization.

- Supports classless routing.

R2

router eigrp 100

network 192.168.2.0 0.0.0.255

network 192.168.4.0 0.0.0.255

eigrp router-id 2.2.2.2

no auto-summary

R3

router eigrp 100

network 192.168.3.0 0.0.0.255

network 192.168.5.0 0.0.0.255

eigrp router-id 3.3.3.3

no auto-summary

R4

router eigrp 100

network 192.168.4.0 0.0.0.255

network 192.168.5.0 0.0.0.255

network 192.168.6.0 0.0.0.255

eigrp router-id 4.4.4.4

no auto-summary

Step 4: Verify Neighbor Relationships

Command:

show ip eigrp neighbors

Example on R1:

Neighbor Address

192.168.2.2

192.168.3.2

Meaning:

- R1 formed adjacency with R2.

- R1 formed adjacency with R3.

Step 5: Verify Router ID

Command:

show ip protocols

Output:

Routing Protocol is "eigrp 100"

Router ID 1.1.1.1

Step 6: Verify Learned Routes

Command:

show ip route

Output:

D 192.168.4.0/24

D 192.168.5.0/24

D 192.168.6.0/24

Explanation:

D = EIGRP Route

These routes were learned dynamically from neighboring routers.

Step 7: End-to-End Connectivity Test

From PC1:

ping 192.168.6.10

Result:

!!!!!

Successful.

From PC1:

ping 192.168.6.20

Result:

!!!!!

Successful.

How the Packet Travels

When PC1 sends a packet to PC2:

PC1

↓

Switch1

↓

R1

↓

R2 or R3

↓

R4

↓

Switch2

↓

PC2

EIGRP automatically determines the best path based on its metric calculation.

Conclusion

In this lab, EIGRP AS 100 was configured on four routers. Unique Router IDs were assigned to each router (1.1.1.1 through 4.4.4.4). All directly connected networks were advertised using EIGRP, neighbor relationships were successfully formed, and routing information was exchanged dynamically. The routers learned remote networks automatically without static routes. Finally, successful ping tests from the LAN connected to R1 to the LAN connected to R4 confirmed that EIGRP was operating correctly and providing end-to-end connectivity across the network.