🌐 CCNA Lab 12 – Standard ACL with IPv4, DHCP, and EIGRP

📌 Lab Title

Configuring Standard Access Control List (ACL), DHCP, IPv4 Addressing, and EIGRP AS 50

🎯 Lab Objective

The objective of this lab was to configure:

- IPv4 Addressing

- DHCP Server for LAN 1 and LAN 2

- EIGRP AS 50 Dynamic Routing

- Standard ACL to restrict network access

The primary goal achieved in this lab was:

✅ Configure IPv4 Addressing

✅ Configure DHCP Pools for Both LANs

✅ Configure EIGRP AS 50

✅ Verify End-to-End Connectivity

✅ Apply a Standard ACL

✅ Block LAN 1 PC1 from pinging LAN 2 PC2

📖 Introduction to Standard ACL

What is an ACL?

An Access Control List (ACL) is a set of rules that controls which packets are allowed or denied through a router.

ACLs improve network security by filtering traffic based on predefined rules.

What is a Standard ACL?

A Standard ACL filters traffic only by the source IPv4 address.

It does not examine:

- Destination IP Address

- Protocol Type

- Port Numbers

It only checks:

Source IPv4 Address

Standard ACL Number Range

1 – 99

1300 – 1999

ACL Processing Rules

ACLs are processed from top to bottom.

The first matching rule is applied.

If no rule matches, there is an implicit deny at the end.

permit 192.168.1.2

deny any

🛠️ Devices Used

| Device | Quantity |
| --- | --- |
| Cisco 1841 Routers | 2 |
| Cisco 2960 Switches | 2 |
| PCs | 4 |
| Copper Straight-Through Cables | 6 |

📊 Network Topology

EIGRP AS 50

192.168.3.0/24

R1 ---------------- R2

|                  |

|                  |

192.168.1.0/24      192.168.2.0/24

|                  |

SW1                SW2

/   \              /   \

PC1   PC2          PC3   PC4

📋 IP Addressing Scheme

| Network | Address |
| --- | --- |
| LAN 1 | 192.168.1.0/24 |
| LAN 2 | 192.168.2.0/24 |
| R1–R2 Link | 192.168.3.0/24 |

Router Interfaces

R1

| Interface | IP Address |
| --- | --- |
| G0/0 | 192.168.3.1/24 |
| G0/1 | 192.168.1.1/24 |

R2

| Interface | IP Address |
| --- | --- |
| G0/0 | 192.168.3.2/24 |
| G0/1 | 192.168.2.1/24 |

🔹 Step 1: Basic Router Configuration

Configure on both routers:

enable

configure terminal

hostname R1

enable secret cisco123

line console 0

password console123

login

service password-encryption

Repeat on R2.

🔹 Step 2: Configure IPv4 Addresses

R1

interface g0/0

ip address 192.168.3.1 255.255.255.0

no shutdown

interface g0/1

ip address 192.168.1.1 255.255.255.0

no shutdown

R2

interface g0/0

ip address 192.168.3.2 255.255.255.0

no shutdown

interface g0/1

ip address 192.168.2.1 255.255.255.0

no shutdown

🔹 Step 3: Configure DHCP Pool

R1 (LAN 1)

Exclude the gateway address:

ip dhcp excluded-address 192.168.1.1

Create DHCP Pool:

ip dhcp pool LAN1

network 192.168.1.0 255.255.255.0

default-router 192.168.1.1

dns-server 8.8.8.8

domain-name ccnalab.com

R2 (LAN 2)

Exclude the gateway address:

ip dhcp excluded-address 192.168.2.1

Create DHCP Pool:

ip dhcp pool LAN2

network 192.168.2.0 255.255.255.0

default-router 192.168.2.1

dns-server 8.8.8.8

domain-name ccnalab.com

🔹 Step 4: Configure EIGRP AS 50

R1

router eigrp 50

no auto-summary

network 192.168.1.0 0.0.0.255

network 192.168.3.0 0.0.0.255

R2

router eigrp 50

no auto-summary

network 192.168.2.0 0.0.0.255

network 192.168.3.0 0.0.0.255

🔹 Step 5: Verify Connectivity

Before applying the ACL, verify that routing works.

From PC1 (LAN 1):

ping 192.168.2.3

Result:

!!!!!

Successful.

This confirms that EIGRP is exchanging routes correctly.

🔹 Step 6: Configure Standard ACL

The objective is to prevent PC1 (192.168.1.2) from reaching LAN 2, while allowing all other devices to communicate.

On R2:

access-list 10 deny host 192.168.1.2

access-list 10 permit any

🔹 Step 7: Apply the ACL

Apply the ACL close to the destination network.

interface g0/1

ip access-group 10 out

Why Outbound?

The traffic is filtered as it leaves R2 toward LAN 2, preventing packets from PC1 from reaching devices on LAN 2.

🔹 Step 8: Verify the ACL

Display the ACL:

show access-lists

Expected Output:

Standard IP access list 10

deny host 192.168.1.2

permit any

Check the interface:

show ip interface g0/1

Expected Output:

Outgoing access list is 10

🔹 Step 9: Test Connectivity After ACL

From PC1 (192.168.1.2)

ping 192.168.2.3

Expected Result:

Request timed out

❌ Ping blocked.

From PC2 (192.168.1.3)

ping 192.168.2.3

Expected Result:

!!!!!

✅ Ping successful.

From PC3 (LAN 2)

ping 192.168.1.3

Expected Result:

!!!!!

✅ Communication still works for permitted hosts.

🔹 Step 10: Save Configuration

copy running-config startup-config

Run on both routers.

📚 Verification Commands

show ip route

show ip eigrp neighbors

show ip protocols

show access-lists

show ip interface g0/1

show ip dhcp binding

show ip dhcp pool

ping

🎓 Concepts Learned

✅ IPv4 Address Configuration

✅ DHCP Server Configuration

✅ DHCP Address Allocation

✅ EIGRP AS 50 Configuration

✅ Standard ACL Creation

✅ Applying ACLs to Interfaces

✅ ACL Rule Processing

✅ Traffic Filtering Based on Source IP

✅ End-to-End Connectivity Verification

📝 Lab Summary

In this lab, two Cisco routers were configured with IPv4 addressing, DHCP services, and EIGRP AS 50 to provide dynamic routing between two LANs. After confirming successful connectivity, a Standard ACL was created on R2 to deny traffic originating from PC1 (192.168.1.2) while allowing all other traffic. The ACL was applied outbound on the interface connected to LAN 2. After implementation, PC1 in LAN 1 was no longer able to ping PC4 (192.168.2.3) in LAN 2, while other hosts continued to communicate successfully. This demonstrated how a Standard ACL can be used to control network access based solely on the source IPv4 address.