🌐 CCNA Lab 07 – OSPF Routing Configuration

📌 Lab Title

Configuration of OSPF Process ID 100 in Area 0

🎯 Lab Objective

The objective of this lab was to configure Open Shortest Path First (OSPF) as a dynamic routing protocol on four routers using Process ID 100 and Area 0.

The main goal achieved in this lab was:

✅ Successfully establish OSPF neighbor relationships between routers.

✅ Dynamically advertise all networks.

✅ Learn remote routes automatically.

✅ Successfully ping from LAN 1 (PC1) to LAN 2 (PC2).

📖 Introduction to OSPF

What is OSPF?

OSPF (Open Shortest Path First) is a Link-State Routing Protocol used to exchange routing information between routers within an Autonomous System.

Unlike RIP, which uses hop count, OSPF uses Cost as its metric to determine the best path.

OSPF is one of the most widely used routing protocols in enterprise networks because it is fast, scalable, and efficient.

Why Use OSPF?

Suppose a company has multiple routers:

R1 ----- R2

|         |

|         |

R3 ----- R4

Instead of manually configuring routes on every router, OSPF allows routers to:

- Automatically discover neighbors.

- Exchange routing information.

- Calculate the best path.

- Adapt to topology changes automatically.

OSPF Features

1. Link-State Protocol

Every router maintains a complete map of the network.

2. Fast Convergence

When a link fails, OSPF quickly recalculates routes.

3. Uses SPF Algorithm

SPF stands for:

Shortest Path First

Developed by:

Dijkstra Algorithm

4. Supports Large Networks

OSPF scales much better than RIP.

5. Uses Areas

OSPF divides networks into logical areas.

The backbone area is:

Area 0

OSPF Terminology

Neighbor

Routers running OSPF that exchange Hello packets.

Example:

R1 -------- R2

Link State Advertisement (LSA)

Information exchanged between routers describing networks and links.

Cost

Metric used by OSPF.

Lower Cost = Better Route

Area

A logical grouping of routers.

Example:

Area 0

All routers in this lab belong to Area 0.

🛠️ Devices Used

| Device | Quantity |
| --- | --- |
| Cisco 1841 Router | 4 |
| Cisco 2960 Switch | 2 |
| PCs | 4 |
| Serial Cables | 4 |
| Copper Straight Through Cables | 6 |

📊 Network Topology

R2

192.168.2.0/24

/        \

/          \

/            \

/              \

R1                R4

\              /

\            /

\          /

\        /

192.168.3.0/24

R3

LAN 1                                 LAN 2

192.168.1.0/24                    192.168.6.0/24

📋 IP Addressing Scheme

Router R1

| Interface | IP Address |
| --- | --- |
| Fa0/0 | 192.168.1.1/24 |
| S0/0/0 | 192.168.2.1/24 |
| S0/0/1 | 192.168.3.1/24 |

Router R2

| Interface | IP Address |
| --- | --- |
| S0/0/0 | 192.168.2.2/24 |
| S0/0/1 | 192.168.4.1/24 |

Router R3

| Interface | IP Address |
| --- | --- |
| S0/0/0 | 192.168.3.2/24 |
| S0/0/1 | 192.168.5.1/24 |

Router R4

| Interface | IP Address |
| --- | --- |
| Fa0/0 | 192.168.6.1/24 |
| S0/0/0 | 192.168.5.2/24 |
| S0/0/1 | 192.168.4.2/24 |

🖥️ PC Configuration

LAN 1

PC1

IP Address : 192.168.1.10

Subnet Mask: 255.255.255.0

Default Gateway: 192.168.1.1

PC2

IP Address : 192.168.1.20

Subnet Mask: 255.255.255.0

Default Gateway: 192.168.1.1

LAN 2

PC3

IP Address : 192.168.6.10

Subnet Mask: 255.255.255.0

Default Gateway: 192.168.6.1

PC4

IP Address : 192.168.6.20

Subnet Mask: 255.255.255.0

Default Gateway: 192.168.6.1

🔹 Step 1: Basic Router Configuration

Performed on all routers:

enable

configure terminal

hostname R1

enable secret cisco123

line console 0

password console123

login

exit

service password-encryption

🔹 Step 2: Configure IP Addresses

Configured all FastEthernet and Serial interfaces according to the IP addressing table.

Example on R1:

interface fa0/0

ip address 192.168.1.1 255.255.255.0

no shutdown

interface s0/0/0

ip address 192.168.2.1 255.255.255.0

clock rate 64000

no shutdown

interface s0/0/1

ip address 192.168.3.1 255.255.255.0

clock rate 64000

no shutdown

🔹 Step 3: Configure OSPF Process 100

R1

router ospf 100

network 192.168.1.0 0.0.0.255 area 0

network 192.168.2.0 0.0.0.255 area 0

network 192.168.3.0 0.0.0.255 area 0

R2

router ospf 100

network 192.168.2.0 0.0.0.255 area 0

network 192.168.4.0 0.0.0.255 area 0

R3

router ospf 100

network 192.168.3.0 0.0.0.255 area 0

network 192.168.5.0 0.0.0.255 area 0

R4

router ospf 100

network 192.168.4.0 0.0.0.255 area 0

network 192.168.5.0 0.0.0.255 area 0

network 192.168.6.0 0.0.0.255 area 0

🔹 Step 4: Verify OSPF Neighbors

Command:

show ip ospf neighbor

Expected Result on R1:

Neighbor ID     State

2.2.2.2         FULL

3.3.3.3         FULL

This indicates successful OSPF adjacency formation.

🔹 Step 5: Verify OSPF Routes

Command:

show ip route ospf

Expected Output:

O 192.168.4.0/24

O 192.168.5.0/24

O 192.168.6.0/24

Meaning

O = OSPF Learned Route

🔹 Step 6: Verify OSPF Database

show ip ospf database

Purpose:

- Displays LSAs learned from neighboring routers.

- Confirms successful OSPF information exchange.

🔹 Step 7: Test Connectivity

From PC1 (LAN 1)

ping 192.168.6.10

Result:

!!!!!

Successful.

Ping Another Host in LAN 2

ping 192.168.6.20

Result:

!!!!!

Successful.

🔹 Step 8: Save Configuration

On all routers:

copy running-config startup-config

📚 Verification Commands Used

show ip ospf neighbor

show ip route

show ip route ospf

show ip ospf database

show running-config

ping

🎓 Concepts Learned

✅ OSPF Dynamic Routing

✅ OSPF Process ID Configuration

✅ Area 0 Configuration

✅ OSPF Neighbor Adjacency

✅ Link-State Advertisements (LSAs)

✅ SPF Algorithm

✅ OSPF Route Learning

✅ Routing Table Verification

✅ End-to-End Connectivity Testing

📝 Lab Summary

In this lab, OSPF Process ID 100 was configured on four routers using a multi-path topology. All interfaces were assigned IP addresses and advertised into Area 0 using OSPF network statements. OSPF neighbor relationships were successfully established between routers, and routing information was exchanged dynamically through LSAs. The routing tables showed OSPF-learned routes marked with O, confirming successful route propagation. The primary objective of the lab was achieved when PC1 in LAN 1 (192.168.1.10) successfully pinged PC3 in LAN 2 (192.168.6.10), demonstrating complete end-to-end connectivity through OSPF dynamic routing.