🌐 CCNA Lab 02 – Recursive Static Routing Configuration

📌 Lab Title

Configuration of Recursive Static Routing Between Two Routers

🎯 Lab Objective

The purpose of this lab is to:

- Configure basic settings on Cisco routers.

- Assign IP addresses to router interfaces.

- Connect two different LAN networks through routers.

- Configure Recursive Static Routes.

- Verify end-to-end connectivity between networks.

- Understand how routers use next-hop IP addresses to reach remote networks.

🛠️ Devices Used

| Device | Quantity |
| --- | --- |
| Cisco Router (1841/2911) | 2 |
| Cisco Switch | 2 |
| PC | 4 |
| Copper Straight Through Cable | 6 |
| Serial Cable | 1 |

📋 IP Addressing Scheme

Router R1

| Interface | IP Address | Subnet Mask |
| --- | --- | --- |
| Fa0/0 | 192.168.1.1 | 255.255.255.0 |
| S0/0/0 | 192.168.3.1 | 255.255.255.252 |

Router R2

| Interface | IP Address | Subnet Mask |
| --- | --- | --- |
| Fa0/0 | 192.168.2.1 | 255.255.255.0 |
| S0/0/0 | 192.168.3.2 | 255.255.255.252 |

PCs

LAN 1

| Device | IP Address | Gateway |
| --- | --- | --- |
| PC1 | 192.168.1.10 | 192.168.1.1 |
| PC2 | 192.168.1.20 | 192.168.1.1 |

LAN 2

| Device | IP Address | Gateway |
| --- | --- | --- |
| PC3 | 192.168.2.10 | 192.168.2.1 |
| PC4 | 192.168.2.20 | 192.168.2.1 |

🔹 Step 1: Build the Topology

Add Devices

✔ 2 Routers

✔ 2 Switches

✔ 4 PCs

🔹 Step 2: Basic Configuration on R1

Enter CLI:

enable

configure terminal

Change Hostname

hostname R1

Secure Router

enable secret cisco123

line console 0

password console123

login

exit

line vty 0 4

password vty123

login

exit

service password-encryption

Save Configuration

end

copy running-config startup-config

🔹 Step 3: Configure Interfaces on R1

configure terminal

interface fa0/0

ip address 192.168.1.1 255.255.255.0

no shutdown

exit

Configure Serial Interface

interface s0/0/0

ip address 192.168.3.1 255.255.255.252

clock rate 64000

no shutdown

exit

🔹 Step 4: Basic Configuration on R2

enable

configure terminal

hostname R2

enable secret cisco123

line console 0

password console123

login

exit

line vty 0 4

password vty123

login

exit

service password-encryption

🔹 Step 5: Configure Interfaces on R2

LAN Interface

interface fa0/0

ip address 192.168.2.1 255.255.255.0

no shutdown

exit

Serial Interface

interface s0/0/0

ip address 192.168.3.2 255.255.255.252

no shutdown

exit

🔹 Step 6: Configure PC Addresses

PC1

IP Address : 192.168.1.10

Subnet Mask: 255.255.255.0

Gateway    : 192.168.1.1

PC2

IP Address : 192.168.1.20

Subnet Mask: 255.255.255.0

Gateway    : 192.168.1.1

PC3

IP Address : 192.168.2.10

Subnet Mask: 255.255.255.0

Gateway    : 192.168.2.1

PC4

IP Address : 192.168.2.20

Subnet Mask: 255.255.255.0

Gateway    : 192.168.2.1

🔹 Step 7: Configure Recursive Static Route

What is Recursive Static Routing?

A Recursive Static Route uses the next-hop IP address instead of the exit interface.

The router must perform an additional lookup (recursive lookup) to determine which interface should be used to reach the next-hop address.

Configure Route on R1

R1 must reach network:

192.168.2.0/24

through next-hop:

192.168.3.2

Command:

ip route 192.168.2.0 255.255.255.0 192.168.3.2

Configure Route on R2

R2 must reach network:

192.168.1.0/24

through next-hop:

192.168.3.1

Command:

ip route 192.168.1.0 255.255.255.0 192.168.3.1

🔹 Step 8: Verify Routing Table

On R1

show ip route

Output:

S    192.168.2.0/24 [1/0] via 192.168.3.2

On R2

show ip route

Output:

S    192.168.1.0/24 [1/0] via 192.168.3.1

🔹 Step 9: Verify Connectivity

Test Serial Link

From R1:

ping 192.168.3.2

Expected:

!!!!!

Test End-to-End Connectivity

From PC1:

ping 192.168.2.10

Expected:

Reply from 192.168.2.10

Additional Tests

ping 192.168.2.20

ping 192.168.1.10

ping 192.168.1.20

All pings should succeed.

🔹 Step 10: Save Configuration

On R1

copy running-config startup-config

On R2

copy running-config startup-config

📚 Commands Used in This Lab

enable

configure terminal

hostname

enable secret

line console 0

line vty 0 4

password

login

service password-encryption

interface

ip address

no shutdown

clock rate

ip route

show ip route

ping

copy running-config startup-config

🎓 Concepts Learned

✅ Router Basic Configuration

✅ Hostname Configuration

✅ Interface Configuration

✅ Serial Link Configuration

✅ Static IP Addressing

✅ Default Gateway Configuration

✅ Recursive Static Routing

✅ Route Table Verification

✅ Connectivity Testing

✅ Network Troubleshooting

📝 Lab Summary

In this lab, two routers, two switches, and four PCs were connected to create two separate LANs. Basic router security settings such as hostname, console password, VTY password, and enable secret were configured. IP addresses were assigned to all interfaces and end devices. A point-to-point serial connection using the 192.168.3.0/30 network was established between R1 and R2. Recursive static routes were then configured using the next-hop IP addresses (192.168.3.1 and 192.168.3.2) to enable communication between the 192.168.1.0/24 and 192.168.2.0/24 networks. Connectivity was verified successfully using ping commands, demonstrating proper implementation of recursive static routing.