🌐 CCNA Lab 09 – Route Redistribution Between RIP and EIGRP

📌 Lab Title

Redistribution Between RIP and EIGRP Routing Protocols

🎯 Lab Objective

The objective of this lab was to configure Route Redistribution between two different routing protocols:

- RIP Version 2

- EIGRP AS 50

The network was divided into two routing domains:

RIP Domain

R1 ←→ R2

EIGRP Domain

R2 ←→ R3

Router R2 acted as the Redistribution Router (Boundary Router) between RIP and EIGRP.

The main goal achieved in this lab was:

✅ Configure RIP Version 2

✅ Configure EIGRP AS 50

✅ Configure Route Redistribution

✅ Exchange Routes Between RIP and EIGRP

✅ Verify Learned Routes

✅ Successfully Ping Between Different LANs

📖 Introduction to Route Redistribution

What is Route Redistribution?

Route Redistribution is the process of sharing routes between different routing protocols.

For example:

RIP Network

↓

Redistribution Router

↓

EIGRP Network

Without redistribution:

- RIP routers know only RIP routes.

- EIGRP routers know only EIGRP routes.

With redistribution:

- RIP routes become visible to EIGRP routers.

- EIGRP routes become visible to RIP routers.

Why Use Redistribution?

Large organizations often have different routing protocols running in different parts of the network.

Example:

Branch Office → RIP

Head Office → EIGRP

To allow communication between both networks, redistribution is required.

🛠️ Devices Used

| Device | Quantity |
| --- | --- |
| Cisco 2901 Router | 3 |
| Cisco 2960 Switch | 2 |
| PC | 2 |
| Copper Straight Through Cable | 4 |

📊 Network Topology

RIP DOMAIN

PC1

|

SW1

|

R1 -------- R2 -------- R3

192.168.1.0   192.168.2.0

EIGRP AS 50

|

SW2

|

PC2

📋 IP Addressing Scheme

LAN 1

| Device | IP Address |
| --- | --- |
| PC1 | 192.168.3.2 |
| R1 G0/1 | 192.168.3.1 |

Network:

192.168.3.0/24

R1 – R2 Link

| Device | IP Address |
| --- | --- |
| R1 G0/0 | 192.168.1.1 |
| R2 G0/0 | 192.168.1.2 |

Network:

192.168.1.0/24

R2 – R3 Link

| Device | IP Address |
| --- | --- |
| R2 G0/1 | 192.168.2.1 |
| R3 G0/0 | 192.168.2.2 |

Network:

192.168.2.0/24

LAN 2

| Device | IP Address |
| --- | --- |
| PC2 | 192.168.4.2 |
| R3 G0/1 | 192.168.4.1 |

Network:

192.168.4.0/24

🔹 Step 1: Basic Router Configuration

Perform on all routers.

Example R1

enable

configure terminal

hostname R1

enable secret cisco123

line console 0

password console123

login

service password-encryption

Repeat for R2 and R3.

🔹 Step 2: Configure Interfaces

R1

interface g0/0

ip address 192.168.1.1 255.255.255.0

no shutdown

interface g0/1

ip address 192.168.3.1 255.255.255.0

no shutdown

R2

interface g0/0

ip address 192.168.1.2 255.255.255.0

no shutdown

interface g0/1

ip address 192.168.2.1 255.255.255.0

no shutdown

R3

interface g0/0

ip address 192.168.2.2 255.255.255.0

no shutdown

interface g0/1

ip address 192.168.4.1 255.255.255.0

no shutdown

🔹 Step 3: Configure RIP Version 2

R1

router rip

version 2

no auto-summary

network 192.168.1.0

network 192.168.3.0

R2 (RIP Side)

router rip

version 2

no auto-summary

network 192.168.1.0

🔹 Step 4: Configure EIGRP AS 50

R3

router eigrp 50

no auto-summary

network 192.168.2.0 0.0.0.255

network 192.168.4.0 0.0.0.255

R2 (EIGRP Side)

router eigrp 50

no auto-summary

network 192.168.2.0 0.0.0.255

🔹 Step 5: Configure Route Redistribution on R2

R2 is connected to both routing domains.

Redistribute RIP into EIGRP

router eigrp 50

redistribute rip metric 10000 100 255 1 1500

Metric Meaning

Bandwidth  = 10000

Delay      = 100

Reliability= 255

Load       = 1

MTU        = 1500

These values are required because EIGRP needs a metric when importing routes.

Redistribute EIGRP into RIP

router rip

redistribute eigrp 50 metric 2

Explanation

Metric 2 = Hop Count

RIP requires hop count information.

🔹 Step 6: Verify Routing Tables

On R1

show ip route

Expected:

R 192.168.2.0/24

R 192.168.4.0/24

These routes came from EIGRP through redistribution.

On R3

show ip route

Expected:

D EX 192.168.1.0/24

D EX 192.168.3.0/24

Meaning

D EX = External EIGRP Route

These routes originated from RIP.

🔹 Step 7: Verify RIP Routes

show ip route rip

Expected:

R 192.168.4.0/24

🔹 Step 8: Verify EIGRP External Routes

show ip route eigrp

Expected:

D EX 192.168.3.0/24

🔹 Step 9: Test Connectivity

From PC1 (RIP Network)

ping 192.168.4.2

Result:

!!!!!

Successful.

From PC2 (EIGRP Network)

ping 192.168.3.2

Result:

!!!!!

Successful.

Packet Flow

PC1 (192.168.3.2)

│

▼

R1 (RIP)

│

▼

R2 (Redistribution Router)

│

▼

R3 (EIGRP)

│

▼

PC2 (192.168.4.2)

🔹 Step 10: Save Configuration

copy running-config startup-config

Run on all routers.

📚 Verification Commands

show ip route

show ip route rip

show ip route eigrp

show ip protocols

show running-config

ping

🎓 Concepts Learned

✅ RIP Version 2 Configuration

✅ EIGRP AS 50 Configuration

✅ Route Redistribution

✅ External EIGRP Routes (D EX)

✅ RIP Route Exchange

✅ Redistribution Metrics

✅ Boundary Router Configuration

✅ Dynamic Route Sharing

✅ End-to-End Connectivity Testing

📝 Lab Summary

In this lab, two different routing protocols (RIP Version 2 and EIGRP AS 50) were configured in separate routing domains. Router R2 served as the redistribution router and was configured to exchange routing information between both protocols. RIP routes were redistributed into EIGRP, and EIGRP routes were redistributed into RIP using appropriate metrics. After verifying the routing tables, redistributed routes appeared successfully on both sides of the network. The lab objective was achieved when PC1 (192.168.3.2) in the RIP network successfully pinged PC2 (192.168.4.2) in the EIGRP network, proving successful route redistribution and end-to-end connectivity between different routing protocols.